# 低速无人车线控底盘生产下线检测报告

# 1. 检测对象与结论

| 会话 ID | EOL-0B96CC1113B6 |
| --- | --- |
| 底盘编号 | PH3-NORMAL-001 |
| VIN | PH3NORMAL0000001 |
| 序列号 | PH3-1783678136013 |
| 工位 | EOL-STATION-01 |
| 操作员 | dev-operator |
| 开始时间 | 2026-07-10T10:08:56.032852+00:00 |
| 结束时间 | - |
| 检测结论 | PASS |
| 失败原因 | - |

# 2. 追溯信息

| software_version | v1.0.2 |
| --- | --- |
| dbc_hash | 387ae48bd84852c8 |
| config_hash | profile:dev |
| config_profile | dev |
| test_plan_id | default_chassis_eol_v1 |
| test_plan_version | 2.0.0 |
| operator | dev-operator |

# 3. 检测步骤

| 序号 | 步骤 | 状态 | 结果 | 耗时(ms) | 失败原因 |
| --- | --- | --- | --- | --- | --- |
| 1 | 上电自检 | DONE | PASS | 108 | - |
| 2 | CAN 通信检测 | DONE | PASS | 202 | - |
| 3 | BMS 检测 | DONE | PASS | 253 | - |
| 4 | VCU 状态检测 | DONE | PASS | 260 | - |
| 5 | 电机心跳检测 | DONE | PASS | 206 | - |
| 6 | 档位检测 | DONE | PASS | 1594 | - |
| 7 | 低速驱动检测 | DONE | PASS | 2497 | - |
| 8 | 转向检测 | DONE | PASS | 4055 | - |
| 9 | 灯光检测 | DONE | PASS | 4391 | - |
| 10 | 制动停止检测 | DONE | PASS | 2600 | - |
| 11 | 告警复查 | DONE | PASS | 202 | - |
| 12 | 报告生成 | DONE | PASS | - | - |

# 4. 检测断言

| 步骤 | 断言 | 信号 | 阈值 | 测量值 | 单位 | 质量/来源 | 结果 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 上电自检 | 数据库可写 | system | true | true | - | good / | PASS |
| 上电自检 | DBC 已加载 | system | true | true | - | good / | PASS |
| 上电自检 | 急停已释放 | system | true | true | - | good / | PASS |
| CAN 通信检测 | CAN1 原始接收在线 | system | true | true | - | good / | PASS |
| CAN 通信检测 | CAN2 原始接收在线 | system | true | true | - | good / | PASS |
| CAN 通信检测 | 关键报文完整 | 0x51,0x77,0x168,0xE1 | ["0x168", "0x51", "0x77", "0xE1"] | {"present": ["0x168", "0x51", "0x77", "0xE1"], "missing": []} | - | good / | PASS |
| BMS 检测 | BMS 总压在范围内 | BMS_Voltage | {"min": 36.0, "max": 72.0, "note": "site_confirm_battery_pack_voltage"} | 52.0 | V | good / 0x100 | PASS |
| BMS 检测 | BMS SOC 不低于阈值 | BMS_SOC | 30 | 86 | % | good / 0x100 | PASS |
| BMS 检测 | NTC 温度均在范围内 | NTC1,NTC2,NTC3 | {"min": -20, "max": 60} | [25, 26, 24] | degC | good / 0x103 | PASS |
| BMS 检测 | 0x102 保护位均未触发 | 0x102 | 0 | {"BMS_Protect_Bit_00": 0, "BMS_Protect_Bit_01": 0, "BMS_Protect_Bit_02": 0, "BMS_Protect_Bit_03": 0, "BMS_Protect_Bit_04": 0, "BMS_Protect_Bit_05": 0, "BMS_Protect_Bit_06": 0, "BMS_Protect_Bit_07": 0, "BMS_Protect_Bit_08": 0, "BMS_Protect_Bit_09": 0, "BMS_Protect_Bit_10": 0, "BMS_Protect_Bit_11": 0, "BMS_Protect_Bit_12": 0, "BMS_Protect_Bit_13": 0, "BMS_Protect_Bit_14": 0, "BMS_Protect_Bit_15": 0, "BMS_Protect_Short_Circuit": 0, "BMS_Protect_Over_Current": 0, "BMS_Protect_Over_Temperature": 0, "BMS_Protect_Under_Temperature": 0, "BMS_Protect_Over_Voltage": 0, "BMS_Protect_Under_Voltage": 0, "BMS_Protect_MOS_Fault": 0, "BMS_Protect_Precharge_Fault": 0, "BMS_Protect_Bitmap": 0} | bool/bitmap | good / 0x102 | PASS |
| VCU 状态检测 | 点火状态有效 | CCU_Ignition_Status | null | 1 | - | good / 0x51 | PASS |
| VCU 状态检测 | 初始车速接近零 | CCU_Vehicle_Speed | 0.2 | 0.0 | km/h | good / 0x51 | PASS |
| VCU 状态检测 | 无非预期制动冲突 | Remote_Brake_Request_Status,Touch_Brake_Request_Status | null | [false, false] | bool | good / 0x51 | PASS |
| 电机心跳检测 | 双电机心跳在线 | 0x703,0x704 | ["0x703", "0x704"] | {"present": ["0x703", "0x704"], "missing": []} | - | good / | PASS |
| 档位检测 | 档位反馈跟随命令 | CCU_Shift_Level_Status | "feedback follows D/N/R/N" | [{"expected": 0, "actual": 0, "observed": [0.0, 0.0, 0.0, 0.0, 0.0], "matched": true}, {"expected": 1, "actual": 1, "observed": [1.0, 1.0, 1.0, 1.0, 1.0], "matched": true}, {"expected": 3, "actual": 3, "observed": [3.0, 3.0, 3.0, 3.0, 3.0], "matched": true}, {"expected": 0, "actual": 0, "observed": [0.0, 0.0, 0.0, 0.0], "matched": true}] | - | good / | PASS |
| 低速驱动检测 | 车速达到目标附近 | CCU_Vehicle_Speed | {"target": 1.5, "tolerance": 0.5} | [0.0, 0.1, 0.3, 0.4, 0.6, 0.7, 0.9, 1.0, 1.2, 1.3, 1.4, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5, 1.5] | km/h | good / 0x51 | PASS |
| 低速驱动检测 | 四轮轮速一致 | Wheel_Speed_Front_Left_RPM,Wheel_Speed_Front_Right_RPM,Wheel_Speed_Rear_Left_RPM,Wheel_Speed_Rear_Right_RPM | {"max_delta": 30.0} | [[200.0, 200.0, 200.0, 200.0]] | rpm | good / 0x168 | PASS |
| 转向检测 | 前转向反馈跟随命令 | SAS_Front_Angle | {"max_error": 8.0} | [{"expected": 30.0, "actual": 29.9, "error": 0.10000000000000142, "sample_count": 20}, {"expected": -30.0, "actual": -29.9, "error": 0.10000000000000142, "sample_count": 21}, {"expected": 0.0, "actual": 0.0, "error": 0.0, "sample_count": 20}] | cmd | good / 0xE1 | PASS |
| 转向检测 | 后转向反馈跟随命令 | SAS_Rear_Angle | {"max_error": 8.0} | [{"expected": 30.0, "actual": 29.9, "error": 0.10000000000000142, "sample_count": 20}, {"expected": -30.0, "actual": -29.9, "error": 0.10000000000000142, "sample_count": 21}, {"expected": 0.0, "actual": 0.0, "error": 0.0, "sample_count": 20}] | cmd | good / 0xE1 | PASS |
| 灯光检测 | 灯光反馈跟随命令 | Left_Turn_Light_Status,Right_Turn_Light_Status,Position_Light_Status,Low_Beam_Status | "feedback follows light commands" | [{"signal": "Left_Turn_Light_Status", "expected": true, "actual": true, "matched": true}, {"signal": "Right_Turn_Light_Status", "expected": true, "actual": true, "matched": true}, {"signal": "Position_Light_Status", "expected": true, "actual": true, "matched": true}, {"signal": "Low_Beam_Status", "expected": true, "actual": true, "matched": true}] | bool | good / | PASS |
| 制动停止检测 | 制动后车速归零 | CCU_Vehicle_Speed | 0.2 | 0.0 | km/h | good / 0x51 | PASS |
| 制动停止检测 | 制动反馈已激活 | Brake_Status | true | [true, true, true, true, true, true, true, true, true, true, true, true, true, true, true, true, true, true, true, true] | bool | good / 0x51 | PASS |
| 告警复查 | 最高告警等级为零 | 0x77 | 0 | 0 | - | good / 0x77 | PASS |
| 告警复查 | 无关键通道超时 | can_statistics | "all channels healthy" | {"unhealthy_channels": []} | - | good / | PASS |

# 5. 结论与签名

检测结论：PASS；通过步骤 12/12；失败断言 0/24。

操作员签名：________________    质量工程师：________________